PCB Construction – July 11 2019

- Source Article http://www.technoblogy.com/show?2FCL - Includes a through hole PCB for panel mounting.

- Reference files on GitHub - https://github.com/JohnGENZ/ZL3TILProject

- Circuit Diagram

- Performance

- Our Board is for chassis mounting and as we design it here it is single sided with no holes

- The double sided version.

- CAD packages and SMD components.

CAD is quick, and more and more components are only available as SMD parts.  (Some SMD component can be purchased on breakout boards – at a cost.) https://www.sparkfun.com/products/10124

CAD software - Eagle, KiCAD, Altrium, OrCAD, PADs ……

Eagle well supported by Open Source Community –  Adafruit, SparkFun make Eagle drawings available.  Supported by the major parts suppliers e.g. Mouser, Digikey.

- Eagle is found here:  https://www.autodesk.com/products/eagle/free-download

2 layer Eagle is free – board size and other limitations apply – but probably good enough for us.  Fabrication services also have very low prices for “small” boards – so it costs a lot more if you need to go larger.  2 layer boards can also be a good choice for prototyping because it is easier to get probes to all parts of the circuit.

Other versions are compared here: https://www.autodesk.com/products/eagle/compare

- Eagle looks like this:   ……

- Workflow:

  - Choose key components

  - Create a schematic drawing

  - Layout the PCB

  - Generate a Bill of Materials

  - Make the Board

  - Assemble the board.

- Selecting Components:

  - There are parts libraries available from different places:

    - Adafruit.  https://github.com/adafruit/Adafruit-Eagle-Library

    - Sparkfun https://github.com/sparkfun/SparkFun-Eagle-Libraries

    - Suppliers, eg Mouser https://nz.mouser.com/ProductDetail/Wurth-Elektronik/885012207098?qs=%2Fha2pyFaduhWLwq%252BImdxjPxGLxfkz82R3MUHb0DTRlyTmZ0GqCfc7w%3D%3D

    - Assemblers, eg Seeed https://www.seeedstudio.com/opl.html

  - Check availability of components – capacitors and some connectors are hard to get at the moment.

- Create a schematic drawing:

This tutorial covers the basis well.  https://learn.sparkfun.com/tutorials/using-eagle-schematic/all

  - Start Eagle – you want to start a new project.

  - After creating the project, create a new schematic.

  - Place the components on the drawing.  Use the library manager to browse to the parts libraries and add parts to the schematic

  - V1 – empty

  - V1i – all parts – note that connectors used even if at the end all we want are the solder pads.

  - V1ii – lay out parts

  - V1iii – join the dots

  - V1iiii – all joined up

  - V1iv – prettified

  - V1v – values added, ready for laying out a board.

- Layout the PCB:

Choose the design rules:

- These are the rules that govern how the board can be made.

- Most typically they relate to the capability of the fabricator or the fabrication process.  The answer is different if you want to mill a board, or hand drill it, or it may depend on how you apply the resist.  They may also relate to the electrical properties of the circuit – maybe for current carrying or rf performance.

Eg  http://support.seeedstudio.com/knowledgebase/articles/447362-fusion-pcb-specification

- Don’t use minimum values unless you really must.  6mil spacing and 10 mil traces are very small!  0.5mm is probably the smallest dimension for milling.

- default JGE Seeed V6 10mil Trace free.dru is a modified version of the Eagle default files for use with the free version of Eagle (2 layers).

Make a board

- Click SCH/BRD to generate a blank board.

- Set the size of the board – note that pin spacing is usually .1 of an inch so the grid is good at 0.05 of an inch.  Our board is 1400mil by 1600mil (35.5mm by 40.64mm)

- Put the mounting holes on the board!!! 2.5mm = 98.425 mil

- Load the DRU file!!

- Place the components   Start point, with components available

- V1vi – Components placed – needs ground pour.

- V1vii – Components prettified – ground pour added (Ground pour is a polygon with isolate set to 12mil).

- V1viii – routing complete

- V1vii – Components prettified – ground pour added.

Notes on the auto router

- it should usually complete more than 80% - if not try a different component placing.

- As you get better you will get more “picky” about the auto router – it does an OK job for lower frequencies and good signal levels.

- The autorouter completes from where the board as routed.  Try routing “important” signals manually before running the autorouter.   See https://www.autodesk.com/products/eagle/blog/are-you-better-than-the-autorouter/

- The autorouter sticks to the grid – this can make for some strange results.

- Sometimes the autorouter does not like the ground pour on a single sided board – try removing it (ripping it up) if the autorouter is not working properly.

- Manual routing and connecting up the ground pours is usually obvious – but may not be pretty.  The autorouter is limited in the number of vias and corners a route can have.